The cryptographic algorithm used to encrypt and decrypt files in this setup is AES (Advanced Encryption Standard), operating in CBC (Cipher Block Chaining) mode with PKCS#5 padding. Let’s break down the algorithm and why it’s used:

### 1. Advanced Encryption Standard (AES)

- Definition: AES is a symmetric encryption algorithm, meaning the same key is used for both encryption and decryption. It is widely regarded as secure and efficient for encrypting large amounts of data.

- Key Features:

  - Block Cipher: AES operates on fixed-size blocks of data (16 bytes or 128 bits).

  - Key Lengths: Commonly supports 128, 192, or 256-bit keys. In this case, we are using a 256-bit key for strong security.

  - Efficiency: AES is computationally efficient, making it suitable for file encryption, even for large files.

### 2. Cipher Block Chaining (CBC) Mode

- Definition: CBC is an encryption mode for block ciphers that introduces dependency between plaintext blocks. Each plaintext block is XORed with the previous ciphertext block before being encrypted.

- How it Works:

  - The first plaintext block is XORed with an initialization vector (IV) and then encrypted.

  - Subsequent plaintext blocks are XORed with the previous ciphertext block and then encrypted.

  - Decryption reverses this process, using the same IV and key.

- Why CBC Mode?

  - Chaining Dependency: Ensures that identical plaintext blocks produce different ciphertext, improving security against certain attacks (e.g., pattern recognition).

  - IV Requirement: An IV is used to randomize the encryption for the first block, ensuring that repeated encryptions of the same data result in different ciphertexts.

### 3. PKCS#5 Padding

- Why Padding? AES operates on fixed-size blocks, but plaintext files often don’t align perfectly with the block size. Padding ensures that the last block is the correct size.

- How PKCS#5 Works:

  - If the plaintext is not a multiple of the block size, PKCS#5 adds extra bytes to the last block.

  - The value of the padding byte corresponds to the number of padding bytes added (e.g., if 3 bytes are added, the padding is 0x03 0x03 0x03).

  - During decryption, the padding is removed automatically.

### 4. Key and IV Usage

- Key:

  - The AES key is a 256-bit symmetric key that is generated once per session using a secure random number generator.

  - It is shared between the client and server using RSA encryption during the authentication process.

- IV (Initialization Vector):

  - The IV is a 16-byte random value generated per session.

  - It ensures that the same plaintext encrypted with the same key produces different ciphertext.

### Encryption and Decryption Workflow

- Encryption Process (During sendFile or uploadFile):

  - The plaintext data is read in chunks.

  - Each chunk is:

    - XORed with the IV (first block) or previous ciphertext (subsequent blocks).

    - Encrypted using the AES key in CBC mode.

  - The ciphertext is sent to the recipient.

- Decryption Process (During downloadFile or receiveFile):

  - The ciphertext is received in chunks.

  - Each chunk is:

    - Decrypted using the AES key in CBC mode.

    - XORed with the IV (first block) or previous ciphertext (subsequent blocks) to recover the plaintext.

### Why AES-CBC is a Good Choice

- Security:

  - AES is considered secure against all known attacks when used with a strong key (e.g., 256-bit).

  - CBC mode with a random IV ensures ciphertext variability, even for identical plaintexts.

- Performance:

  - AES is efficient and can encrypt/decrypt large files quickly, making it suitable for file transfer applications.

- Flexibility:

  - CBC mode is compatible with padding schemes like PKCS#5, allowing it to handle plaintext of varying sizes.

### Limitations of AES-CBC

- IV Transmission:

  - The IV must be securely shared with the recipient. In this setup, the IV is encrypted using RSA and sent to the recipient during authentication.

- Sequential Dependency:

  - CBC mode requires sequential processing, which can limit parallelism compared to other modes like GCM (Galois/Counter Mode).

### Conclusion

By using AES-CBC with PKCS#5 padding, this implementation achieves a balance between strong security, performance, and compatibility. The session-specific AES key and IV, securely exchanged using RSA, ensure confidentiality and integrity for the file transfers.